Role: You are an expert Python developer specializing in industrial automation and GUI design using customtkinter and the pycomm3 library.

Objective: Create a professional-grade Python GUI to control a Masterflex REGLO Pump (Model 78018-xx) via EtherNet/IP.

Connection Details:

- Protocol: EtherNet/IP.

- IP Address: [Insert your Pump's IP here].

- Interface: Ethernet cable to PC.

Core Functionality Requirements:

- Connection Management: A sidebar to input the IP address and a "Connect/Disconnect" status indicator.

- Operational Controls:

  - Start/Stop/Pause: Toggle buttons that write to Byte 0, Bits 0 & 1.

  - Direction: A switch to toggle between CW and CCW (Byte 0, Bit 6).

  - Mode Selection: Radio buttons to switch between Continuous, Time, and Volume modes (Byte 4).

- Parameter Setting (Input):

  - Flow Rate: A slider and numerical entry box (Float) writing to Bytes 8-11.

  - Dispense Volume: Input field for Volume Mode (Float) writing to Bytes 12-15.

  - Batch Total: Input field for the number of cycles (Bytes 24-27).

- Real-Time Monitoring (Feedback):

  - Cumulative Volume: Display a running total from Bytes 8-11 (Input Data).

  - Current Flow Rate: Display live feedback from Bytes 36-39 (Input Data).

  - Pump Status: A "Traffic Light" indicator using bits from Bytes 0-3 (e.g., Bit 1: Running, Bit 4: Head Open).

- Safety & Errors:

  - Display a text console that decodes Error Codes from the pump (e.g., Error #6: Head Open, Error #7: Over Temp).

UI Style: Use a dark-themed, modern dashboard aesthetic with "Masterflex Orange" accents.

Why this works

- Direct Register Access: By telling the AI to use pycomm3 and providing the specific byte offsets from your manual (e.g., Bytes 8-11 for flow rate), you ensure the Python script talks directly to the pump's brain.

- Feedback Loops: The prompt includes "Input Data" (pump to PC) and "Output Data" (PC to pump), allowing the GUI to show you what the pump is actually doing in real-time.

- Error Handling: Including the error definitions from Section 5 ensures your GUI doesn't just "crash" if something goes wrong; it will tell you "Pump Head Opened" or "Motor Over Speed".

Vital Setup Steps Before Running the Code

- Enable the Protocol: On the pump touchscreen, navigate to Settings and ensure EtherNet/IP is toggled to ON.

- Static IP: In Ethernet Settings, switch from DHCP to STATIC and assign an IP (e.g., 192.168.0.50) so the Python script can find it every time.

- Install Libraries: You will need to run pip install pycomm3 customtkinter on your computer.